|  | Main engine | # A41S | 1 | 72000 | AKASAKA | CCS certificate |
| --- | --- | --- | --- | --- | --- | --- |
|  | 主机 |  |  |  | 日本赤坂 | According to |
|  | Type: vertical, direct injection, 4-stroke, |  |  |  |  | technical agreement |
|  | with turbocharger and air cooler, closed |  |  |  |  | 按技术协议 |
|  | water cooling, non-reversible, marine |  |  |  |  |  |
|  | low speed diesel engine. |  |  |  |  |  |
|  | Cyl. No. : 6 |  |  |  |  |  |
|  | Bore & stroke: 410×800 |  |  |  |  |  |
|  | Rated output: 2647kW |  |  |  |  |  |
|  | Rated revolution: 240 r/min |  |  |  |  |  |
|  | Oil sump: dry type |  |  |  |  |  |
|  | Direction of engine rotation (Ahead): |  |  |  |  |  |
|  | Clockwise (view from flywheel) |  |  |  |  |  |
|  | Starting mode: Compressed air |  |  |  |  |  |
|  | SFOC: 181g/kW·h |  |  |  |  |  |
|  | Fuel oil: HFO 180cst / 50C |  |  |  |  |  |
|  | Attachment supplied with engine: |  |  |  |  |  |
|  | Air. cooler |  |  |  |  |  |
|  | F.O. filter |  |  |  |  |  |
|  | L.O. filter |  |  |  |  |  |
|  | Tools, spare parts etc. |  |  |  |  |  |
|  | (Refer to M.E. specification) |  |  |  |  |  |
|  | Reduction gear box | # GWC63.71 | 1 | 15000 | CHONGQING GEARBOX Co., Ltd. | CCS certificate |
|  | 减速齿轮箱 |  |  |  | 重庆齿轮箱责任有限公司 | According to |
|  | Max. revolution: 240 r/min |  |  |  |  | technical agreement |
|  | Reduction ratio: 2.05：1 |  |  |  |  | 按技术协议 |
|  | Deliver ability: 13.462kW/r.min-1 |  |  |  |  |  |
|  | Model of main engine: A41S |  |  |  |  |  |
|  | Direction of input shaft rotation: |  |  |  |  |  |
|  | Clockwise (view from output end) |  |  |  |  |  |
|  | Input coupling type: |  |  |  |  |  |
|  | High-flexible coupling |  |  |  |  |  |
|  | Type of operation: |  |  |  |  |  |
|  | Electric pneumatic operated |  |  |  |  |  |
|  | 带PTO |  |  |  |  |  |
|  | High-flexible coupling |  | 1 | ~700 | Provided by the gearbox manufacturer | CCS certificate |
|  | 高弹性联轴器 |  |  |  | 齿轮箱厂家配套 | According to |
|  | Rated torque: 160kNm |  |  |  |  | technical agreement |
|  | Max allowable torque: 400kNm |  |  |  |  | 按技术协议 |
|  | Model of main engine: A41S |  |  |  |  |  |
|  | Model of reduction gear box: GWC63.71 |  |  |  |  |  |
|  | Auxiliary engine |  |  |  |  |  |
|  | 辅机 |  |  |  |  |  |
| 4.1 | Diesel generator set | CCFJ400J | 3 | ~20700 | Zhejiang Lianda Power Equipment Co., Ltd. | CCS certificate |
|  | 主柴油发电机组 |  |  |  | 浙江联大动力设备有限公司 | According to |
|  | Engine : | K19-DM |  |  | Cummins | technical agreement |
|  | Rated output: 485kW |  |  |  | 康明斯 | 按技术协议 |
|  | Rated revolution: 1800r/min |  |  |  |  |  |
|  | Starting mode: air motor |  |  |  |  |  |
|  | Cooling mode: closed water cooling |  |  |  |  |  |
|  | Generator : | MP-H-400-6 |  |  | Kangfu generator 康富电机 |  |
|  | Rated output: 400kW |  |  |  | SHANGHAI MARATHON-GEXIN ELECTRIC |  |
|  | Rated voltage: 450V |  |  |  | CO., LTD上海马拉松革新电气有限公司 |  |
|  | Frequency: 60 Hz |  |  |  |  |  |
|  | Cooling mode: air cooling |  |  |  |  |  |
|  | Power factor: cosφ = 0.8 (lag) |  |  |  |  |  |
|  | Wire connection: 3 phase |  |  |  |  |  |
|  | Excitation mode: brushless |  |  |  |  |  |
|  | Attachment: |  |  |  |  |  |
|  | spare parts, special use tools, float tank， |  |  |  |  |  |
|  | self-starting device etc. |  |  |  |  |  |
| 4.2 | Emergency diesel generator set | CCFJ75Y | 1 | ~1000 | Zhejiang Lianda Power Equipment Co., Ltd. | CCS certificate |
|  | 应急发电机组 |  |  |  | 浙江联大动力设备有限公司 |  |
|  | Engine: | 6BT5.9-GM |  |  | Cummins |  |
|  | Rated output: 100 kW |  |  |  | 康明斯 |  |
|  | Rated revolution: 1800r/min |  |  |  |  |  |
|  | Starting mode: electric motor / spring electric motor |  |  |  |  |  |
|  | Cooling mode: fan air cooling |  |  |  |  |  |
|  | Generator : | MP-H-75-4 |  |  | Kangfu generator 康富电机 |  |
|  | Rated output: 75kW |  |  |  | SHANGHAI MARATHON-GEXIN ELECTRIC |  |
|  | Rated voltage: 450V |  |  |  | CO., LTD上海马拉松革新电气有限公司 |  |
|  | Frequency: 60 Hz |  |  |  |  |  |
|  | Power factor: cosφ = 0.8 (lag) |  |  |  |  |  |
|  | Wire connection: 3 phase |  |  |  |  |  |
|  | Excitation mode: brushless |  |  |  |  |  |
|  | Attachment: |  |  |  |  |  |
|  | spare parts, special use tools, self-starting |  |  |  |  |  |
|  | device( including starting control box, battery ) etc. |  |  |  |  |  |
| 4.3 | Shaft driven generator |  |  |  | Dalian CNISEGG Electric Co., Ltd |  |
|  | 轴带发电机 |  | 1 |  | 大连珂尼赛格电气有限公司 | CCS certificate |
|  | Rated output: 300kW |  |  |  |  |  |
|  | Rated revolution: 1800r/min |  |  |  |  |  |
|  | Cooling mode: air cooling |  |  |  |  |  |
|  | Boiler |  |  |  |  |  |
|  | 锅炉 |  |  |  |  |  |
| 5.1 | Exhaust thermal oil heater | # QFK65 | 1 |  |  | CCS certificate |
|  | 废气热油锅炉 |  |  |  |  | According to |
|  | Type: vertical, exh. gas heating |  |  |  | JIANGYIN SANJIE INDUSTRY CO.,LTD | technical agreement |
|  | Hot area: ~65 m2 |  |  |  | 江阴三杰实业有限公司 | 按技术协议 |
|  | Working pressure: 0.7MPa |  |  |  |  |  |
|  | Attachment: |  |  |  |  |  |
|  | 1)Control box, |  | 1 |  |  |  |
|  | 2)Boiler feed water pump: |  | 2 |  |  |  |
| 5.2 | Thermal medium heater | # QXC-250L | 2 | ~18000 |  | CCS certificate |
|  | 货油加热用导热油锅炉 |  |  |  |  | According to |
|  | Type: vertical |  |  |  | JIANGYIN SANJIE INDUSTRY CO.,LTD | technical agreement |
|  | Heating power: 3000kW |  |  |  | 江阴三杰实业有限公司 | 按技术协议 |
|  | Design pressure: 1.0 MPa |  |  |  |  | Symmetrical arranged |
|  | Working pressure: 0.7 MPa |  |  |  |  | 左右对称布置 |
|  | Working medium: thermal oil |  |  |  |  | Weishaupt burner |
|  | Electric power: ~60 kW |  |  |  |  | 德国威索燃烧器 |
|  | F.O. consumption: 300kg/h（H.F.O 180cst/ 50C） |  |  |  |  |  |
|  | Attachment: |  |  |  |  |  |
|  | 1)Control box |  |  |  |  |  |
|  | 2)Expansion tank 2.5m3 |  | 1 |  |  |  |
|  | 3)Oil gas separator SJFL-150 |  | 1 |  |  |  |
|  | 4)Heat oil circulating pump | WRY100-65-240 | 3 |  |  |  |
|  | Capacity： 120m3/h |  |  |  |  |  |
|  | Pressure： 0.7MPa |  |  |  |  |  |
|  | Electric motor： 30kW AC440V |  |  |  |  |  |
|  | 5)Heat oil filling pump | 2CY3.3/3.3-1 | 1 |  |  |  |
|  | Capacity： 3.3m3/h |  |  |  |  |  |
|  | Pressure： 0.32MPa |  |  |  |  |  |
|  | Electric motor： 2.2kW AC440V |  |  |  |  |  |
|  | 6)Boiler fuel supply pump set: |  | 2 |  |  |  |
|  | Electric motor： 0.75kW AC440V |  |  |  |  |  |
|  | 7)MGO cooler |  | 2 |  |  |  |
|  | Purifier |  |  |  |  | CCS certificate |
|  | 分油机 |  |  |  |  |  |
| 6.1 | F.O. purifier unit | HM605-A2 | 1 |  |  |  |
|  | 燃油分油机单元 |  |  |  | Alfa Laval Shanghai |  |
|  | Purifier model : | KYDH204SD-23 | 2 |  | 上海阿法拉伐技术有限公司 |  |
|  | Type: self-cleaning，partial discharge |  |  |  |  |  |
|  | Capacity: ~1300 L/h （H.F.O 180cst/ 50C） |  |  |  |  |  |
|  | Heating mode: thermal oil |  |  |  |  |  |
|  | Electric motor: 2.2kW AC 440V |  |  |  |  |  |
|  | Attachment: thermal oil heater, feed pump (1.1kW), |  |  |  |  |  |
|  | electric control box etc. |  |  |  |  |  |
| 6.1 | D.O. purifier unit |  |  |  |  |  |
|  | 轻油分油机单元 | KYDH204SD-23 | 1 |  |  | CCS certificate |
|  | Purifier model : |  |  |  |  |  |
|  | Type: self-cleaning，partial discharge |  |  |  |  |  |
|  | Capacity: ~1300 L/h （D.O） |  |  |  |  |  |
|  | Electric motor: 2.2kW AC 440V |  |  |  |  |  |
|  | Attachment :, feed pump (1.1kW), |  |  |  |  |  |
|  | electric control box etc. |  |  |  |  |  |
| 6.3 | L.O. purifier unit | HM615-A2 | 1 |  |  | CCS certificate |
|  | 滑油分油机单元 |  |  |  |  |  |
|  | Purifier model: | KYDH204SD-23 | 2 |  |  |  |
|  | Discharge mode: |  |  |  |  |  |
|  | Capacity: ＞1500 L/h |  |  |  |  |  |
|  | Electric motor: 3kW AC 440V |  |  |  |  |  |
|  | Attachment: electric heater, feed pump (1.1kW), |  |  |  |  |  |
|  | electric control box etc. |  |  |  |  |  |
|  | ME F.O. supply unit | HY-01BSS | 1 | ~2000 | Zhejiang Huayi Ship Equipment | CCS certificate |
|  | 主机供油单元 |  |  |  | Manufacture Co., Ltd. |  |
|  | Fuel oil: H.F.O. 380cst/50C |  |  |  | 浙江华益船舶设备制造有限公司 |  |
|  | Rated F.O. supply: 1.2 m3/h |  |  |  |  |  |
|  | Circulating capacity: 1.5 m3/h |  |  |  |  |  |
|  | Pressure of F.O. outlet: 0. 49MPa |  |  |  |  |  |
|  | Filter fineness: 10 μm |  |  |  |  |  |
|  | Heating medium: electric/thermal oil |  |  |  |  |  |
|  | Power : ~4 kW AC440V |  |  |  |  |  |
|  | Air compressor set |  |  |  |  | CCS certificate |
|  | 空气压缩机组 |  |  |  |  |  |
| 8.1 | Main Air compressor set | CVF-60/30 | 3 | 580 | 日本株式会社 田邊空気機械製作所 |  |
|  | 空气压缩机组 |  |  |  | TANABE PNEUMATIC MACHINERY CO. |  |
|  | Type: water cooling, double stage, piston, |  |  |  |  |  |
|  | self control |  |  |  |  |  |
|  | Capacity: 60m3/h |  |  |  |  |  |
|  | Pressure: 3.0MPa |  |  |  |  |  |
|  | Electric motor: ~15kW AC 440V |  |  |  |  |  |
| 8.2 | Emgergencfy Air compressor set | CVF-20/30 | 1 |  |  |  |
|  | 应急压缩机组 |  |  |  |  |  |
|  | Type: air cooling, double stage, piston, |  |  |  |  |  |
|  | self control |  |  |  |  |  |
|  | Capacity: 20m3/h |  |  |  |  |  |
|  | Pressure: 3.0MPa |  |  |  |  |  |
|  | Electric motor: 5.5kW AC 440V |  |  |  |  |  |
|  | Controlling air dryer |  | 1 |  |  |  |
|  | 控制空气干燥器 |  |  |  |  |  |
|  | Type: freeze drying |  |  |  |  |  |
|  | Pressure & temperature of air intake: |  |  |  |  |  |
|  | ≤1MPa,≤55℃ |  |  |  |  |  |
|  | Capacity of treat: 50 m3/h |  |  |  |  |  |
|  | Power: 0.52kW AC220V/50Hz/1Φ |  |  |  |  |  |
|  | Fans |  |  |  |  |  |
|  | 风机 |  |  |  |  |  |
| 10.1 | ER fan | JCZ-90B | 2 | 824 |  | both to |
|  | 机舱通风机 |  |  |  | Qingdao Yongjian Marine Machinery | be reversible |
|  | Type: Marine axial flow fan |  |  |  | Manufacture Co., Ltd. | 两条台可逆转 |
|  | Capacity: 42000m3/h |  |  |  | 青岛永建船舶机械制造有限公司 |  |
|  | Total pressure: 860Pa |  |  |  |  |  |
|  | Electric motor: 18.5kW AC 440V | Y160M-4H |  |  |  |  |
| 10.2 | Cargo oil pump room fan | CBZ-70B II | 3 | 654 |  |  |
|  | 货油泵舱风机 |  |  |  |  | 一抽两送 |
|  | Type: Marine explosion-proof axial flow fan |  |  |  |  |  |
|  | Capacity: 18000m3/h |  |  |  |  |  |
|  | Total pressure: 300Pa |  |  |  |  |  |
|  | Electric motor: 3kW AC 440V | YB100L2-4-H |  |  |  |  |
| 10.3 | Cargo area void exhaust fan | CBZ-70A II | 2 | 438 |  |  |
|  | 货舱隔离空舱抽风机 |  |  |  |  |  |
|  | Type: Marine explosion-proof axial flow fan |  |  |  |  |  |
|  | Capacity: 18000m3/h |  |  |  |  |  |
|  | Total pressure: 500Pa |  |  |  |  |  |
|  | Electric motor: 4kW AC 440V | YB112M-4-H |  |  |  |  |
| 10.4 | Compartment fan | CBZ-30BⅡ | 2 | 132 |  | 二氧化碳室 |
|  | 舱室通风机 |  |  |  |  | 舵机舱送风 |
|  | Type: Marine explosion-proof axial flow fan |  |  |  |  |  |
|  | Capacity: 1200m3/h |  |  |  |  |  |
|  | Total pressure: 330Pa |  |  |  |  |  |
|  | Electric motor: 0.75kW AC 440V | YB80L1-2H |  |  |  |  |
| 10.5 | Compartment fan | CBZ-35BⅡ | 1 | 66 |  | 空调机室抽风 |
|  | 舱室通风机 |  |  |  |  |  |
|  | Type: Marine explosion-proof axial flow fan |  |  |  |  |  |
|  | Capacity: 2100m3/h |  |  |  |  |  |
|  | Total pressure: 510Pa |  |  |  |  |  |
|  | Electric motor: 1.1kW AC 440V | YB80L2-2H |  |  |  |  |
| 10.6 | Compartment fan | CBZ-35AⅡ | 1 | 66 |  | 首侧推舱通风 |
|  | 舱室通风机 |  |  |  |  |  |
|  | Type: Marine explosion-proof axial flow fan |  |  |  |  |  |
|  | Capacity: 5400m3/h |  |  |  |  |  |
|  | Total pressure: 340Pa |  |  |  |  |  |
|  | Electric motor: 1.1kW AC 440V | YB80L2-2H |  |  |  |  |
| 10.7 | Compartment fan | JCZ-25 | 1 | 26 |  | 厨房送风 |
|  | 舱室通风机 |  |  |  |  |  |
|  | Type: Marine axial flow fan |  |  |  |  |  |
|  | Capacity: 1500m3/h |  |  |  |  |  |
|  | Total pressure: 200Pa |  |  |  |  |  |
|  | Electric motor: 0.55kW AC 440V | Y712-2-H |  |  |  |  |
| 10.8 | Compartment fan | JCZ-30A | 2 | 26 |  | 厨房抽风 |
|  | 舱室通风机 |  |  |  |  | 应急发电机室送风 |
|  | Type: Marine axial flow fan |  |  |  |  |  |
|  | Capacity: 1800m3/h |  |  |  |  |  |
|  | Total pressure: 350Pa |  |  |  |  |  |
|  | Electric motor: 0.75kW AC 440V | Y80L1-2-H |  |  |  |  |
| 10.9 | Compartment fan | JCZ-25 | 2 | 26 |  |  |
|  | 舱室通风机 |  |  |  |  | 卫生间抽风 |
|  | Type: Marine axial flow fan |  |  |  |  |  |
|  | Capacity: 1000m3/h |  |  |  |  |  |
|  | Total pressure: 220Pa |  |  |  |  |  |
|  | Electric motor: 0.55kW AC 440V | Y712-2-H |  |  |  |  |
| 10.10 | Compartment fan | CWZ-200DⅡ | 1 | 28.6 |  | 医务室卫生间 |
|  | 舱室通风机 |  |  |  |  | 抽风 |
|  | Type: Marine axial flow fan |  |  |  |  |  |
|  | Capacity: 550m3/h |  |  |  |  |  |
|  | Total pressure: 50Pa |  |  |  |  |  |
|  | Electric motor: 0.06kW AC 440V | Y561-4-H |  |  |  |  |
| 10.11 | Compartment fan | CBZ-30BⅡ | 1 | 79 |  | 油漆间抽风 |
|  | 舱室通风机 |  |  |  |  |  |
|  | Type: Marine explosion-proof axial flow fan |  |  |  |  |  |
|  | Capacity: 1200m3/h |  |  |  |  |  |
|  | Total pressure: 330Pa |  |  |  |  |  |
|  | Electric motor: 0.75kW AC 440V | YB80L1-2H |  |  |  |  |
| 10.12 | Compartment fan | JCZ-20 | 1 | 26 |  |  |
|  | 舱室通风机 |  |  |  |  | 货油泵变频器间抽风 |
|  | Type: Marine axial flow fan |  |  |  |  |  |
|  | Capacity: 800m3/h |  |  |  |  |  |
|  | Total pressure: 160Pa |  |  |  |  |  |
|  | Electric motor: 0.55kW AC 440V | Y712-2-H |  |  |  |  |
|  | Air reservoirs & pressure vessels |  |  |  |  |  |
|  | 空气瓶和压力容器 |  |  |  |  |  |
| 11.1 | Starting air reservoir | A1.0-3 CB/T493-1998 | 2 | 550 |  | CCS certificate |
|  | 起动空气瓶 |  |  |  |  | 或主机配套 |
|  | Cubage: 1.0 m3 |  |  |  |  |  |
|  | Working pressure: 3.0MPa |  |  |  |  |  |
| 11.2 | Emergency starting air reservoir | A0.08-3 CB/T493-1998 | 1 | 120 |  | CCS certificate |
|  | for AE |  |  |  |  |  |
|  | 辅机应急起动空气瓶 |  |  |  |  |  |
|  | Cubage: 0.08 m3 |  |  |  |  |  |
|  | Working pressure: 3.0MPa |  |  |  |  |  |
| 11.3 | Service air reservoir | A0.16-1 CB/T493-1998 | 1 | 155 |  | CCS certificate |
|  | 杂用空气瓶 |  |  |  |  |  |
|  | Cubage: 0.16 m3 |  |  |  |  |  |
|  | Working pressure: 1.0MPa |  |  |  |  |  |
| 11.4 | Control air reservoir | A0.08-1 CB/T493-1998 | 1 | 120 |  |  |
|  | 控制空气瓶 |  |  |  |  |  |
|  | Cubage: 0.08 m3 |  |  |  |  |  |
|  | Working pressure: 1.0MPa |  |  |  |  |  |
| 11.5 | Air reservoir for cargo piping stripping | A1.6-3 CB/T493-1998 | 1 | 950 |  | CCS certificate |
|  | 液货管扫线用空气瓶 |  |  |  |  |  |
|  | Cubage: 1.6 m3 |  |  |  |  |  |
|  | Working pressure: 3.0MPa |  |  |  |  |  |
| 11.6 | F.W. pressure tank | YLG-C0.5 | 1 | 284 | Wuhan Jinding Electromechanical | CCS certificate |
|  | 淡水压力柜 |  |  |  | Equipment Co., Ltd |  |
|  | Capacity: 0.5m3 |  |  |  | 武汉金鼎机电设备有限公司 |  |
|  | Working pressure: 0.4MPa |  |  |  |  |  |
| 11.7 | S.W. pressure tank | YLG-C0.5 |  |  |  | CCS certificate |
|  | 海水压力柜 |  |  |  |  |  |
|  | Capacity: 0.5m3 |  |  |  |  |  |
|  | Working pressure: 0.4MPa |  |  |  |  |  |
| 11.8 | Electric heating hot water tank | ZDR-0.3 | 1 | 330 | Wuhan Jinding Electromechanical | CCS certificate |
|  | 电加热热水柜 |  |  |  | Equipment Co., Ltd |  |
|  | Capacity: 0.3m3 |  |  |  | 武汉金鼎机电设备有限公司 |  |
|  | Working pressure: 0.4MPa |  |  |  |  |  |
|  | Heating mode: Elec. |  |  |  |  |  |
|  | Pressure of steam: 0.3MPa |  |  |  |  |  |
|  | Power source: 18kW AC 440V 50Hz |  |  |  |  |  |
| 11.9 | Anti-osmotic F.W. generator | HSRO-05 | 1 |  | Wuhan Jinding Electromechanical |  |
|  | 反渗透海水淡化装置 |  |  |  | Equipment Co., Ltd |  |
|  | Capacity: 5 t/d |  |  |  | 武汉金鼎机电设备有限公司 |  |
|  | Power: 3.5 kW |  |  |  |  |  |
| 11.10 | Ultraviolet Radiation sterilizer |  | 1 |  | Wuhan Jinding Electromechanical |  |
|  | /Mineralizing device |  |  |  | Equipment Co., Ltd |  |
|  | 紫外线消毒器及矿化装置 |  |  |  | 武汉金鼎机电设备有限公司 |  |
|  | Oil pumps |  |  |  |  |  |
|  | 油泵 |  |  |  |  |  |
| 12.1 | Cargo oil pump | CJB-500CB | 2 | ~3000 | TAIKO KIKAI | CCS certificate |
|  | 货油泵 |  |  |  |  | According to |
|  | Type: vertical, twin screw, with heating chamber |  |  |  |  | technical agreement |
|  | Capacity: 500m3/h |  |  |  |  | 按技术协议 |
|  | Pressure: 0.93MPa |  |  |  |  | 配防爆电机 |
|  | Electric motor: ~330kW×1200r/min |  |  |  |  |  |
| 12.2 | ME L.O. standby pump | YCB50-0.5 | 1 |  |  | CCS certificate |
|  | 主机滑油备用泵 |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Type: horizontal, gear |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Capacity : 50 m3/h |  |  |  |  |  |
|  | Pressure: 0.5MPa |  |  |  |  |  |
|  | Electric motor: 22 kW AC 440V | Y200L2-6H |  |  |  |  |
| 12.3 | L.O. stand-by pump for gear box | 2CY-21/2.5 | 1 |  | Zhejiang Baiyun pump Co., Ltd. | supplied with |
|  | 齿轮箱滑油备用泵 |  |  |  | 浙江白云泵业有限公司 | gear box |
|  | Type: horizontal, gear |  |  |  | Yongjia Huanggong Machinery Co., Ltd. | 齿轮箱配套提供 |
|  | Capacity : 21 m3/h |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Pressure: 2.5 MPa |  |  |  |  |  |
|  | Electric motor: 22 kW AC 440V | Y200L2-6H |  |  |  |  |
| 12.4 | F.O. transfer pump | KCB-135 | 2 |  | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 燃油输送泵 |  |  |  | 浙江白云泵业有限公司 |  |
|  | Type: horizontal, gear |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Capacity: 8m3/h |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Pressure: 0.33MPa |  |  |  |  |  |
|  | Electric motor: 2.2kW AC440V | Y112M-6H |  |  |  |  |
| 12.5 | L.O. transfer pump | KCB-135 | 1 |  | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 滑油输送泵 |  |  |  | 浙江白云泵业有限公司 |  |
|  | Type: horizontal, gear |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Capacity: 8m3/h |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Pressure: 0.33MPa |  |  |  |  |  |
|  | Electric motor: 2.2kW AC440V | Y112M-6H |  |  |  |  |
| 12.6 | Resid. oil pump | 2GM98-25A | 1 |  | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 油渣泵 |  |  |  | 浙江白云泵业有限公司 |  |
|  | Type: horizontal, twin screw |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Capacity: 8m3/h |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Pressure: 0.4MPa |  |  |  |  |  |
|  | Electric motor: 5.5kW AC440V | Y132S-4H |  |  |  |  |
| 12.7 | CYL L.O. transfer pump | KCB-135 | 1 |  | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 气缸油输送泵 |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Type: horizontal, gear |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Capacity: 8m3/h |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Pressure: 0.33MPa |  |  |  |  |  |
|  | Electric motor: 2.2kW AC440V | Y112M-6H |  |  |  |  |
| 12.8 | CYL L.O. handpump | CS-32 CB*165-86 CB*165-86 | 1 |  |  | CCS certificate |
|  | 气缸油手摇泵 |  |  |  |  |  |
|  | Water pumps |  |  |  |  |  |
|  | 水泵 |  |  |  |  |  |
| 13.1 | ME S.W. cooling pump | EMC-150MCT | 2 | ~500 | TAIKO KIKAI | CCS certificate |
|  | 主机海水冷却泵 |  |  |  | BRONZE CASING |  |
|  | Type: vertical, centrifugal |  |  |  |  |  |
|  | Capacity: 125m3/h |  |  |  |  |  |
|  | Pressure: 0.25MPa |  |  |  |  |  |
|  | Electric motor: 15 kW AC440V |  |  |  |  |  |
| 13.2 | ME F.W. cooling pump | EMC-100MCT MC100-100MCT | 2 | ~500 | TAIKO KIKAI | CCS certificate |
|  | 主机淡水冷却泵 |  |  |  | BRONZE CASING |  |
|  | Type: vertical, centrifugal |  |  |  |  |  |
|  | Capacity: 55m3/h |  |  |  |  |  |
|  | Pressure: 0.25MPa |  |  |  |  |  |
|  | Electric motor: 7.5kW AC440V |  |  |  |  |  |
| 13.3 | AE S.W. cooling pump | EMC-100MCT | 2 | ~144 | TAIKO KIKAI | CCS certificate |
|  | 副机海水冷却泵 |  |  |  | BRONZE CASING |  |
|  | Type: vertical, centrifugal |  |  |  |  |  |
|  | Capacity: 65m3/h |  |  |  |  |  |
|  | Pressure: 0.28MPa |  |  |  |  |  |
|  | Electric motor: 11kW AC440V |  |  |  |  |  |
| 13.4 | Packaged air conditioning plant pump | CISG65-160 | 1 | ~72 | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 组装式空调冷却泵 |  |  |  | 浙江白云泵业有限公司 |  |
|  | Type: vertical, centrifugal |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Capacity: 30m3/h |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Pressure: 0.28MPa |  |  |  |  |  |
|  | Electric motor: 4kW AC440V |  |  |  |  |  |
| 13.5 | Cabinet air conditioner cooling pump | CISG40-160(I) | 2 | 39 | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 立柜式空调冷却泵 |  |  |  | 浙江白云泵业有限公司 |  |
|  | （兼冷藏装置冷却泵） |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Type: vertical, centrifugal |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Capacity: 8.8m3/h |  |  |  |  |  |
|  | Pressure: 0.33MPa |  |  |  |  |  |
|  | Electric motor: 3kW AC440V |  |  |  |  |  |
| 13.6 | Assistant S.W. cooling standby pump | EMC-125MCT | 2 | ~450 | TAIKO KIKAI | CCS certificate |
|  | 辅海水冷却备用泵 |  |  |  | BRONZE CASING |  |
|  | Type: vertical, centrifugal |  |  |  |  |  |
|  | Capacity: 80m3/h |  |  |  |  |  |
|  | Pressure: 0.28MPa |  |  |  |  |  |
|  | Electric motor: 15kW AC440V |  |  |  |  |  |
| 13.7 | Fire pump (also for deck foam system) | CLH80-200 | 1 | ~300 | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 消防泵（兼甲板泡沫灭火供水泵） |  |  |  | 浙江白云泵业有限公司 |  |
|  | Type: vertical, centrifugal |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Capacity: 65m3/h |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Pressure: 0.72MPa |  |  |  |  |  |
|  | Electric motor: 30kW AC440V | Y200L1-2H |  |  |  |  |
| 13.8 | General pump (with self-priming device) | CLH80-200 | 1 | ~300 | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | (also for fire, deck foam, bilge system) |  |  |  | 浙江白云泵业有限公司 |  |
|  | 总用泵(带自吸装置) |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | （兼消防、泡沫、舱底泵） |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Type: vertical, centrifugal |  |  |  |  |  |
|  | Capacity: 65m3/h |  |  |  |  |  |
|  | Pressure: 0.72MPa |  |  |  |  |  |
|  | Electric motor: 30kW AC440V | Y200L1-2H |  |  |  |  |
| 13.9 | Bilge pump（with self-priming device） | CLH80-160B | 1 | ~300 | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 舱底泵(带自吸装置) |  |  |  | 浙江白云泵业有限公司 |  |
|  | Type: vertical, centrifugal |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Capacity: 65m3/h |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Pressure: 0.28MPa |  |  |  |  |  |
|  | Electric motor: 11kW AC440V | Y160M1-2H |  |  |  |  |
| 13.10 | Special ballast pump | CLH150-250A | 2 | ~528 | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 专用压载泵 |  |  |  | 浙江白云泵业有限公司 |  |
|  | Type: vertical, self-priming centrifugal |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Capacity: 250m3/h |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Pressure: 0.22MPa |  |  |  |  |  |
|  | Electric motor: 30kW AC440V | # Y200L-4H |  |  |  |  |
| 13.11 | Emergency fire pump |  | 1 | 253 | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 应急消防泵 | 80CBZ-65 |  |  | 浙江白云泵业有限公司 |  |
|  | Type: horizontal, self-priming centrifugal |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Capacity: 50m3/h |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Pressure: 0.60MPa |  |  |  |  |  |
|  | Electric motor: 18.5kW AC440V | Y160L1-2H |  |  |  |  |
| 13.12 | Service bilge pump | 32CBZ-21 | 1 | 50 | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 日用舱底泵 |  |  |  | 浙江白云泵业有限公司 |  |
|  | Type: horizontal, self-priming centrifugal |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Capacity: 8m3/h |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Pressure: 0.21MPa |  |  |  |  |  |
|  | Electric motor: 1.5kW AC440V | Y90S-2H |  |  |  |  |
| 13.13 | Hot water circulation pump | 1.5CWX-2 | 1 | 45 | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 热水循环泵 |  |  |  | 浙江白云泵业有限公司 | or supplied with |
|  | Type: horizontal, vortex pump |  |  |  | Yongjia Huanggong Machinery Co., Ltd. | hot water tank |
|  | Capacity: 3 m3/h |  |  |  | 永嘉黄工机械有限公司 | 或由热水柜配套 |
|  | Pressure: 0.40MPa |  |  |  |  |  |
|  | Electric motor: 2.2kW AC440V | Y90L-2H |  |  |  |  |
| 13.14 | Daily S.W. pump | 1.5CWX-2 | 2 | 135 | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 日用海水泵 |  |  |  | 浙江白云泵业有限公司 | or supplied with |
|  | Type: horizontal, vortex pump |  |  |  | Yongjia Huanggong Machinery Co., Ltd. | pressure tank |
|  | Capacity: 3 m3/h |  |  |  | 永嘉黄工机械有限公司 | 或由压力水柜配套 |
|  | Pressure: 0.40MPa |  |  |  |  |  |
|  | Electric motor: 2.2kW AC440V | Y90L-2H |  |  |  |  |
| 13.15 | Daily F.W. pump | 1.5CWX-2 | 2 | 135 | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 日用淡水泵 |  |  |  | 浙江白云泵业有限公司 | or supplied with |
|  | Type: horizontal, vortex pump |  |  |  | Yongjia Huanggong Machinery Co., Ltd. | pressure tank |
|  | Capacity: 3 m3/h |  |  |  | 永嘉黄工机械有限公司 | 或由压力水柜配套 |
|  | Pressure: 0.40MPa |  |  |  |  |  |
|  | Electric motor: 2.2kW AC440V | Y112M-2H |  |  |  |  |
| 13.16 | Sewage pump | 1PWF-25 | 1 | ~35 | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 生活污水泵 |  |  |  | 浙江白云泵业有限公司 |  |
|  | Type: horizontal, gear pump |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Capacity: 10 m3/h |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Pressure: 0.25MPa |  |  |  |  |  |
|  | Electric motor: 4kW AC440V | Y90L-2H |  |  |  |  |
| 13.17 | Fore ejector bilge pump | CP15-0.5T | 1 | 25.5 | Zhejiang Baiyun pump Co., Ltd. |  |
|  | 艏部喷射舱底泵 |  |  |  | 浙江白云泵业有限公司 |  |
|  | Capacity: 15m3 /h |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Pressure: 0.5 MPa |  |  |  | 永嘉黄工机械有限公司 |  |
| 13.18 | Pump room ejector bilge pump | CP15-0.5T | 1 | 25.5 | Zhejiang Baiyun pump Co., Ltd. |  |
|  | 货泵舱喷射舱底泵 |  |  |  | 浙江白云泵业有限公司 |  |
|  | Capacity: 15m3 /h |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Pressure: 0.5 MPa |  |  |  | 永嘉黄工机械有限公司 |  |
| 13.19 | Cargo oil pump gearbox cooling pump | CISG32-125 | 2 |  | Zhejiang Baiyun pump Co., Ltd. | CCS certificate |
|  | 货油泵齿轮箱冷却泵 |  |  |  | 浙江白云泵业有限公司 |  |
|  | Type: vertical, centrifugal |  |  |  | Yongjia Huanggong Machinery Co., Ltd. |  |
|  | Capacity: 3.5m3/h |  |  |  | 永嘉黄工机械有限公司 |  |
|  | Pressure: 0.22MPa |  |  |  |  |  |
|  | Electric motor: 0.75kW AC440V |  |  |  |  |  |
|  | Ballast water treatment |  |  |  | Shanghai Cyeco Environment technology Co., Ltd | CCS certificate |
|  | 压载水处理装置 |  |  |  | 上海船研环保技术有限公司 |  |
|  | Capacity: 300m3/h |  |  |  |  |  |
|  | Power source: 36 kW AC440V |  |  |  |  |  |
|  | Oily water separator | YSZ-1.0 | 1 | 1700 | Wuhan Jinding Electromechanical | CCS certificate |
|  | 油污水分离装置 |  |  |  | Equipment Co., Ltd | Meeting the resolution |
|  | Capacity: 1.0m3/h |  |  |  | 武汉金鼎机电设备有限公司 | of IMO MEPC 227(64) |
|  | Discharging: < 15ppm |  |  |  |  | 满足IMO MEPC.227（64） |
|  | Discharging mode: automatic & manual |  |  |  |  |  |
|  | Power source: 6 kW AC440V |  |  |  |  |  |
|  |  |  |  |  | Wuhan Jinding Electromechanical |  |
|  | Sewage treatment plant | WCBX-25B | 1 | 3280 | Equipment Co., Ltd | CCS certificate |
|  | 生活污水处理装置 |  |  |  | 武汉金鼎机电设备有限公司 | Meeting the resolution |
|  | Capacity: 25 persons |  |  |  |  | of IMO MEPC227(64) |
|  | Quality of effluent: |  |  |  |  | 满足IMO MEPC227（64） |
|  | Suspended solid ≤ 35mg/L |  |  |  |  |  |
|  | biochemical oxygen demand ≤ 25mg/L |  |  |  |  |  |
|  | colichin≤100pc/100mL |  |  |  |  |  |
|  | Power source: （1.5+0.55×2+0.12）kW AC440V |  |  |  |  |  |
|  | Air conditioning & refrigeration equipment |  |  |  |  |  |
|  | 空调及制冷设备 |  |  |  |  |  |
| 17.1 | Marine packaged air conditioning plant | FJKR-105 | 1 | ~1750 |  | CCS certificate |
|  | 船用组装式空调装置 |  |  |  |  |  |
|  | Cooling capacity: 105kW |  |  |  |  |  |
|  | Heating capacity: 116kW |  |  |  |  |  |
|  | Cooling medium: R404A |  |  |  |  |  |
|  | Namely air volume: 9000 m3/h |  |  |  |  |  |
|  | Cooling water capacity: 30m3 |  |  |  |  |  |
|  | Power source: (26+11) kW AC440V |  |  |  |  | CCS certificate |
| 17.2 | Cabinet air conditioner for ER monitoring room | FLD-15A | 1 | 254 | Taixing Qixian Marine Machinery Co., Ltd. |  |
|  | 机舱监视室立柜式空调机 |  |  |  | 泰兴市启娴船用机械有限公司 |  |
|  | Cooling capacity: 17.44kW (15×103kcal/h) |  |  |  |  |  |
|  | Heating capacity: 9kW (7.7×103kcal/h) |  |  |  |  |  |
|  | Cooling medium: R404A |  |  |  |  |  |
|  | Namely air volume: 3000 m3/h |  |  |  |  |  |
|  | Power source: (4.2+0.09×2) kW AC440V |  |  |  |  |  |
| 17.3 | Marine refrigeration plant | CLZ-3 | 1 | ~750 | Taixing Qixian Marine Machinery Co., Ltd. | CCS certificate |
|  | 船用冷藏装置 |  |  |  | 泰兴市启娴船用机械有限公司 |  |
|  | Cooling capacity: 3kW |  |  |  |  |  |
|  | Cooling medium: R404A |  |  |  |  |  |
|  | Power source: 3.9×2 kW AC440V |  |  |  |  |  |
|  | Workshop equipment |  |  |  |  |  |
|  | 机修设备 |  |  |  |  |  |
| 18.1 | Lathe | SDC36B | 1 |  |  |  |
|  | 车床 |  |  |  |  |  |
|  | Max. rotation diameter: Φ360mm |  |  |  |  |  |
|  | Center distance: 750mm |  |  |  |  |  |
|  | Electric motor: 4kW AC440V |  |  |  |  |  |
| 18.2 | Bench drilling machine | Z512 | 1 |  |  |  |
|  | 立式钻床 |  |  |  |  |  |
|  | Drilling capacity : Φ12mm |  |  |  |  |  |
|  | Electric motor: 0.75kW AC440V |  |  |  |  |  |
| 18.3 | Electric grinder | S3ST-200H | 1 |  |  |  |
|  | 电动砂轮机 |  |  |  |  |  |
|  | Stone wheel: 2×Φ200mm |  |  |  |  |  |
|  | Electric motor: 0.5kW AC440V |  |  |  |  |  |
| 18.4 | Table vice | 300 SG90-80 | 1 |  | 300mm JAW |  |
|  | 转盘式台虎钳 |  |  |  |  |  |
|  | Width of vice jaw: 300mm |  |  |  |  |  |
| 18.5 | ER electric crane | DJH2.0-6-9DG | 1 |  | 2 TONS | CCS certificate |
|  | 机舱电动起吊葫芦 |  |  |  |  |  |
|  | Lifting capacity: 2.0t×9m |  |  |  |  |  |
| 18.6 | Handle crane |  | 4 |  |  |  |
|  | 手动起吊葫芦 |  |  |  |  |  |
|  | Lifting capacity: 0.5t |  |  |  |  |  |
|  | Fire extinguishing device |  |  |  | ZhejiangYaning Fire Fighting Equipment Co., Ltd. |  |
|  | 消防设备 |  |  |  | 浙江亚宁消防设备有限公司 |  |
| 19.1 | CO2 bottle group and accessory |  |  |  |  | CCS certificate |
|  | 二氧化碳瓶组及附件 |  |  |  |  |  |
|  | 1)Dual lined CO2 bottle group 68L | D18 CB/T3294-1998 | 1 | 2460 |  |  |
|  | 2)Dual lined CO2 bottle group 68L | D13 CB/T3294-1998 | 1 |  |  |  |
|  | 3)Marine remote releasing station |  | 2 |  |  |  |
|  | 4)ER release valve with signal | B65 CB/T3294-1998 | 1 |  |  |  |
|  | 5)Pump room release valve with signal | A40 CB/T3294-1998 | 1 |  |  |  |
|  | 6)CO2 nozzle | A4-10 CB/T3294-1998 | 15 |  |  |  |
|  | 7)Back pressure valve |  | 1 |  |  |  |
|  | 8)High pressure stop check valve | DN20 PN15 MPa | 1 |  |  |  |
|  | 9)Weighing device | δ2-150 | 1 |  |  |  |
|  | 10)Whistle for CO2 leakiness |  | 1 |  |  |  |
|  | 11)Sound & light alarm |  | 1 |  |  |  |
|  | 12)Pressure signal transmitter |  | 2 |  |  |  |
|  | 13)Specification board |  | 1 |  |  |  |
| 19.2 | Portable CO2 fire extinguisher for paint room | 7kg | 2 |  |  | CCS certificate |
|  | 油漆间手提式二氧化碳灭火器 |  |  |  |  |  |
| 19.3 | ER local water mist system | ZSXZB-40 | 1 |  | ZhejiangYaning Fire Fighting Equipment Co., Ltd. | CCS certificate |
|  | 机舱固定式局部水基灭火装置 |  |  |  | 浙江亚宁消防设备有限公司 |  |
|  | Type: pump type medium pressure water |  |  |  |  |  |
|  | mist fire-extinguishing system |  |  |  |  |  |
|  | Capacity: 80~160L/min |  |  |  |  |  |
|  | Pressure: 3.5MPa |  |  |  |  |  |
|  | Electric power: 7.5kW AC440V |  |  |  |  |  |
| 19.4 | Fixed deck fire extinguishing device |  | 1 |  |  | CCS certificate |
|  | 固定式甲板泡沫灭火装置 |  |  |  |  |  |
|  | 1)Foam applicator | PQZ8 | 5 |  |  |  |
|  | Jet flow: 8 l/s = 480 l/min |  |  |  |  |  |
|  | Jet pressure: 0.5 MPa |  |  |  |  |  |
|  | Range: 22 m |  |  |  |  |  |
|  | 2) Foam cannon | # PP24.C | 5 |  |  |  |
|  | Jet flow： 24 l/s = 1440 l/min |  |  |  |  |  |
|  | Jet pressure： 0.4~0.6 MPa |  |  |  |  |  |
|  | Range： ≥32 m |  |  |  |  |  |
|  | 3)Proportioning mixer | PZH32 | 1 |  |  |  |
|  | Inlet pressure: 0.6~1.2 MPa |  |  |  |  |  |
|  | Outlet pressure: ＜0.3 MPa |  |  |  |  |  |
|  | Output quantity: 4~32 l/s |  |  |  |  |  |
|  | Foam storage tank: 1.5m3 |  |  |  |  |  |
|  | 4)Environmental protection type fluorine albumen foam | YEF3 |  |  |  |  |
|  | Mixed ratio: 3% |  |  |  |  |  |
|  | Foaming times: ≥7.0 |  |  |  |  |  |
| 19.5 | ER wheeled foam fire extinguisher | MPTZ-45 | 1 |  |  | CCS certificate |
|  | 机舱推车式泡沫灭火器 |  |  |  |  |  |
|  | Capacity: 45L |  |  |  |  |  |
| 19.6 | ER wheeled foam fire extinguisher | MPTZ-135 | 1 |  |  | CCS certificate |
|  | 机舱推车式泡沫灭火器 |  |  |  |  |  |
|  | Capacity: 135L |  |  |  |  |  |
| 19.7 | Portable air-foam monitor | PQB8.C | 2 |  |  | CCS certificate |
|  | 手提式空气泡沫枪 |  |  |  |  |  |
|  | Complete parts for each set: |  |  |  |  |  |
|  | Suck type air-foam monitor |  | 2 |  |  |  |
|  | Portable type foam container: 20L |  | 2 |  |  |  |
|  | Foam stand-by container: 20L |  | 2 |  |  |  |
| 19.8 | Portable powder fire extinguisher | MFZ-5 | 10 |  |  | CCS certificate |
|  | 手提式干粉灭火器 |  |  |  |  |  |
|  | Capacity: 5kg |  |  |  |  |  |
| 19.9 | Emergent breathing device | THB/10-Ⅰ | 6 |  |  | CCS certificate |
|  | 紧急逃生呼吸装置 |  |  |  |  |  |
|  | Abidance using time： ≥10min |  |  |  |  |  |
|  | Pneumatic quick-close valve control box | CSKX-70Ⅳ | 1 | 260 | Wuhan Jinding Electromechanical | CCS certificate |
|  | 气动速闭阀组控制箱 |  |  |  | Equipment Co., Ltd |  |
|  | Air reservoir capacity: 70L |  |  |  | 武汉金鼎机电设备有限公司 |  |
|  | Working pressure: 0.7MPa |  |  |  |  |  |
|  | Output passage: 5 |  |  |  |  |  |
|  | Incinerator | HSINC-18 | 1 |  | Hansun Marine Technology Co., Ltd. | CCS certificate |
|  | 焚烧炉 |  |  |  | 上海汉盛船舶技术有限公司 | 配套供应油渣柜 |
|  | Capacity: 210kW(180000 kcal/h) |  |  |  |  |  |
|  | Electric motor： ～7 kW AC380V |  |  |  |  |  |
|  | Exhaust silencer |  |  |  |  | CCS certificate |
|  | 排气消音器 |  |  |  |  |  |
| 22.1 | # Silencer for AE | VK-200 -200 | 3 |  |  |  |
|  | 副机排气消音器 |  |  |  |  |  |
| 22.2 | Silencer for emergency diesel generator set | VK-80 | 1 |  |  |  |
|  | 应急发电机排气消音器 |  |  |  |  |  |
| 22.3 | Spark extinguisher for thermal medium heater | FHX-600 | 2 |  |  |  |
|  | 热油锅炉排气灭火星器 |  |  |  |  |  |
|  | Stern bearings and stern tube sealing arrangement |  |  |  |  |  |
|  | 尾轴承及尾管密封装置 |  |  |  |  |  |
| 23.1 | # Fore bearing of stern shaft |  | 1 |  |  | CCS certificate |
|  | 尾轴前轴承 |  |  |  |  |  |
|  | Material: macromolecule synthetic material |  |  |  |  |  |
| 23.2 | # Aft bearing of stern shaft |  | 1 |  |  | CCS certificate |
|  | 尾轴后轴承 |  |  |  |  |  |
|  | Material: macromolecule synthetic material |  |  |  |  |  |
| 23.3 | # Water lubricated stern tube fore seal | SM-390 | 1 |  |  | CCS certificate |
|  | 水润滑尾轴管前密封装置 |  |  |  |  |  |
|  | Oil discharge monitoring and control system |  | 1 |  |  | CCS certificate |
|  | 排油监控系统 |  |  |  |  | 电气部件防爆 |
|  | # Portable oil/water interface detector |  | 1 |  |  | CCS certificate |
|  | 便携式油水界面探测器 |  |  |  |  |  |
|  | # Portable flammable gas detector |  | 1 |  |  | CCS certificate |
|  | 便携式可燃气体探测仪 |  |  |  |  |  |
|  | # Deck valves DN50 |  | 8 |  |  | CCS certificate |
|  | 甲板阀 |  |  |  |  |  |
|  | # High velocity P/V valve |  | 2 |  |  | CCS certificate |
|  | 高速透气阀 |  |  |  |  |  |
|  | # DN200 |  |  |  |  |  |
|  | # cooler |  |  |  |  | CCS certificate |
|  | 主机冷却器 |  |  |  |  |  |
| 29.1 | # F.W. cooler |  | 1 |  |  | CCS certificate |
|  | 淡水冷却器 |  |  |  |  |  |
|  | Area: 15m² |  |  |  |  |  |
| 29.2 | # L.O. cooler |  | 1 |  |  | CCS certificate |
|  | 滑油冷却器 |  |  |  |  |  |
|  | Area: 42m² |  |  |  |  |  |
| 30 | SCR SYSTEM |  |  |  |  | CCS certificate |
| 30.1 | ME SCR |  | 1 |  |  |  |
|  | 主机催化还原系统 |  |  |  | Nanjing Southeast Industrial Equipment corporation |  |
|  |  |  |  |  | 南京东南工业装备股份有限公司 |  |
| 30.2 | AE SCR |  | 3 |  |  |  |
|  | 辅机催化还原系统 |  |  |  |  |  |